Moment of a force: Part 1

The magnitude of the moment of a force:

MO: Magnitude of the moment of F around point O

d: Perpendicular distance from O to the line of action of F

Note: moving a force along its line of action does not change its moment

Mo = Fd = F rSin (θ)

atau F diurai lebih dahulu menjadi komponen yang  berimpit dengan r  dan komponen yang tegak lurus r.

Komponen yang berimpit dengan r tidak menimbulkan momen terhadap O. Yang menimbulkan momen hanya komponen yang tegak lurus r, sehingga:

Mo = F1 r = FSin(θ) r

Hasilnya sama saja

Direction of the moment in 2-D:

The direction of the moment is given by the right hand rule: Counter Clockwise (CCW) is out of the page, Clockwise (CW) is into the page.

Calculating the moment in 2-D using components: Moments add together as vectors. Select a positive direction (CCW or CW), then calculate each moment and add them using the proper sign for each term. For example:

Mehrdad Negahban and the University of Nebraska, 1996-2006.

All rights reserved

Copy and distribute freely for personal use only

Department of Engineering Mechanics, University of Nebraska, Lincoln, NE 68588-0526