SAFETY INCIDENT REPORT

Report Number: SIR-2024-002

Date of Incident: 8 February 2024

Time of Incident: 14:30

Location: Welding Bay 2, Goldfields Precision Manufacturing

Reported By: Mark Stevens, Production Supervisor

Report Date: 8 February 2024

INJURED PERSON DETAILS:

Name: Robert Chen

Employee ID: EMP-2019-087

Position: Welder Grade 2

Department: Production

Age: 28

Experience: 5 years welding, 2 years with company

INCIDENT DESCRIPTION:

While performing overhead welding on conveyor support structure CS-011, the injured person experienced a minor burn to the left forearm when hot spatter contacted exposed skin between glove and sleeve. The incident occurred during the final cap pass of a fillet weld.

IMMEDIATE ACTIONS TAKEN:

1. Work stopped immediately

2. First aid administered by trained first aider (Jenny Walsh)

3. Burn treated with cold water for 10 minutes

4. Area secured and isolated

5. Supervisor notified within 15 minutes

6. Incident scene photographed

INJURY DETAILS:

Type of Injury: Minor burn (1st degree)

Body Part Affected: Left forearm, 3cm x 1cm area

Medical Treatment: First aid only

Time Lost: 30 minutes

Return to Work: Same day with restrictions

WITNESS STATEMENTS:

Witness 1: Michael Roberts (QC Inspector)

"I saw Robert welding overhead when sparks fell down. He immediately stopped and called for help. He was wearing all required PPE."

Witness 2: Tony Smith (Maintenance Technician)

"I was working nearby and heard Robert call out. The welding area looked normal, nothing unusual about the setup."

ROOT CAUSE ANALYSIS:

Primary Cause: Gap in personal protective equipment coverage

Contributing Factors:

1. Welding jacket sleeve length insufficient for overhead position

2. Work positioning required extended reach

3. Spatter deflection not considered in setup

IMMEDIATE CORRECTIVE ACTIONS:

1. All welders briefed on PPE requirements for overhead work

2. Additional arm protection issued to all welding staff

3. Work method reviewed for overhead welding positions

PREVENTIVE ACTIONS:

1. Update PPE standard to specify minimum sleeve overlap

2. Conduct toolbox talk on overhead welding safety

3. Review all current overhead welding tasks

4. Install additional spatter shields where practical

INVESTIGATION TEAM:

Lead Investigator: David Thompson (Safety Manager)

Team Members: Mark Stevens (Production Supervisor)

Dr. Sarah Wilson (Occupational Health Nurse)

REGULATORY REQUIREMENTS:

WorkSafe WA Notification: Not required (minor injury)

Workers Compensation Claim: No

Medical Clearance Required: No

EQUIPMENT INVOLVED:

Welding Machine: WLD-003 (MIG Welder Station 3)

Last Service Date: 25 January 2024

Condition: Good working order

Safety Systems: All functional

ENVIRONMENTAL CONDITIONS:

Temperature: 32°C

Humidity: 45%

Ventilation: Adequate

Lighting: 800 lux (adequate)

Noise Level: 78 dB

TRAINING RECORDS:

Last Safety Training: 15 November 2023

Welding Certification: Current (expires 30 June 2024)

PPE Training: 10 January 2024

Overhead Work Training: Not completed

FOLLOW-UP ACTIONS:

Action Owner: Mark Stevens

Target Date: 15 February 2024

1. Enrol Robert in overhead welding safety course

2. Conduct workplace assessment of all overhead welding positions

3. Update safe work procedures

APPROVALS:

Safety Manager: David Thompson

Date: 9/02/2024

Signature: D. Thompson

Production Manager: Sarah Chen

Date: 9/02/2024

Signature: S. Chen

General Manager: ________________

Date: ________________

INCIDENT CLOSURE:

All Actions Completed: ☐ YES ☐ NO

Verified By: ________________

Date: ________________

Report Closed By: ________________

Date: ________________

Note: This incident report contains intentional procedural errors for training purposes.