NON-CONFORMANCE REPORT (NCR)

NCR Number: NCR-2024-003

Date Raised: 15 February 2024

Project: PMO-2024-CV-001 - Pilbara Iron Ore Conveyor Components

Raised By: Jennifer Walsh, QC Inspector

Department: Quality Control

DESCRIPTION OF NON-CONFORMANCE:

During routine dimensional inspection of conveyor support structure CS-008, the

following discrepancies were identified:

1. Overall height dimension: 1,847mm (Drawing specification: 1,850mm ± 3mm)

2. Bolt hole centres: 398mm (Drawing specification: 400mm ± 1mm)

3. Weld profile on connection plate exceeds maximum throat thickness by 2mm

DETECTION METHOD:

Routine dimensional inspection using calibrated measuring equipment

- Steel rule (Cal. Cert: SR-2023-089, Due: 15/03/2024) - VALID

- Vernier callipers (Cal. Cert: VC-2023-156, Due: 22/02/2024) - VALID AT TIME OF INSPECTION

- Weld gauge (Cal. Cert: WG-2023-078, Due: 10/04/2024) - VALID

ROOT CAUSE ANALYSIS (5-WHY METHOD):

1. Why did dimensional non-conformances occur?

- Incorrect setup of welding jig during assembly

2. Why was the jig setup incorrect?

- Jig calibration was overdue by 2 weeks

3. Why was jig calibration overdue?

- No systematic calibration management system in place

4. Why is there no systematic calibration management?

- Calibration treated as administrative task rather than critical control

5. Why is calibration not treated as critical control?

- Management system lacks understanding of ISO 9001 Clause 7.1.5 requirements

PRIMARY ROOT CAUSE: Inadequate calibration management system per ISO 9001 Clause 7.1.5

CONTRIBUTING FACTORS:

- Operator training on new jig setup incomplete

- Work instruction WI-2024-CS-008 Rev B not followed correctly

- No verification checks built into assembly process

- Lack of backup coverage for calibration technician

IMMEDIATE ACTION TAKEN:

1. Item CS-008 quarantined in designated area (Red Tag #RT-2024-089)

2. Production stopped on similar items (CS-009 to CS-012)

3. Supervisor notified: Mark Stevens (15/02/2024 at 10:30)

4. Customer notification completed: 15/02/2024 at 14:00 (Email sent to Project Manager)

CUSTOMER NOTIFICATION DETAILS:

Customer: Pilbara Iron Ore Pty Ltd

Contact: John Anderson, Project Manager

Notification Method: Email + Phone call

Date/Time: 15/02/2024 at 14:00

Impact Assessment: 2-day delivery delay, no safety implications

Customer Response: Acknowledged, requested daily updates

CORRECTIVE ACTIONS:

Action 1: Emergency calibration of welding jig

- Responsibility: Quality Manager (David Thompson)

- Target Date: 16/02/2024

- Completion Date: 16/02/2024

- Verification: Calibration certificate CAL-JIG-001-2024

Action 2: Rework CS-008 to correct dimensional issues

- Responsibility: Production Supervisor (Mark Stevens)

- Target Date: 18/02/2024

- Completion Date: 18/02/2024

- Verification: Re-inspection report INS-CS-008-REV1

Action 3: Re-train assembly operators on jig setup procedures

- Responsibility: Production Supervisor (Mark Stevens)

- Target Date: 20/02/2024

- Completion Date: 20/02/2024

- Verification: Training records TR-2024-045 to TR-2024-048

Action 4: Update work instruction to include additional checkpoints

- Responsibility: Quality Manager (David Thompson)

- Target Date: 22/02/2024

- Completion Date: 22/02/2024

- Verification: Work Instruction WI-2024-CS-008 Rev C issued

PREVENTIVE ACTIONS:

Action 1: Implement daily jig verification checks

- Responsibility: QC Inspector (Michael Roberts)

- Target Date: 25/02/2024

- Completion Date: 25/02/2024

- Verification: Daily check sheet DCS-JIG-001 implemented

Action 2: Review and update calibration schedule for all assembly fixtures

- Responsibility: Quality Manager (David Thompson)

- Target Date: 01/03/2024

- Completion Date: 01/03/2024

- Verification: Calibration schedule CAL-SCHED-2024 Rev B

Action 3: Enhance operator training program with competency assessment

- Responsibility: HR Manager + Production Supervisor

- Target Date: 15/03/2024

- Completion Date: 15/03/2024

- Verification: Training matrix TM-2024-001 and competency assessments

Action 4: Install visual aids at workstation

- Responsibility: Production Supervisor (Mark Stevens)

- Target Date: 28/02/2024

- Completion Date: 28/02/2024

- Verification: Visual aid installation VA-CS-008

IMPACT ASSESSMENT:

Schedule Impact: 2 days delay on delivery milestone (recovered through overtime)

Cost Impact: $3,200 (rework labour + materials + overtime costs)

Customer Impact: Delivery delay communicated and accepted

Quality Impact: No safety implications identified

Regulatory Impact: No regulatory notification required

DISPOSITION:

☐ Use as is

☐ Repair

☑ Rework

☐ Scrap

☐ Return to supplier

REWORK DETAILS:

1. Machine bolt holes to correct centres using drill press (Completed 17/02/2024)

2. Grind excess weld material to specification (Completed 17/02/2024)

3. Re-inspect all dimensions per drawing PMO-CV-001-CS-008 Rev C (Completed 18/02/2024)

4. Update inspection records (Completed 18/02/2024)

VERIFICATION OF CORRECTIVE ACTION:

Action Owner: Mark Stevens, Production Supervisor

Target Completion: 20 February 2024

Actual Completion: 20 February 2024

Verification Method: Re-inspection by independent QC inspector (Jennifer Walsh)

Verification Results: All dimensions within specification, weld profile acceptable

EFFECTIVENESS REVIEW:

Review Date: 15/03/2024 (30 days post-closure)

Reviewer: Quality Manager (David Thompson)

Effectiveness Criteria: No similar non-conformances in 30-day period

Results: No similar dimensional non-conformances reported

Additional Actions Required: None

APPROVALS:

Quality Manager: David Thompson

Date: 15/02/2024

Signature: D. Thompson

Production Manager: Sarah Chen

Date: 16/02/2024

Signature: S. Chen

General Manager: [Name Required]

Date: 16/02/2024

Signature: [Signature Required]

CUSTOMER NOTIFICATION RECORD:

Customer Notification Required: YES ☑ NO ☐

Customer Notified By: David Thompson, Quality Manager

Date: 15/02/2024

Method: Email + Phone call

Customer Acknowledgment: Received 15/02/2024 at 15:30

CLOSURE:

Corrective Actions Completed: ☑ YES ☐ NO

Preventive Actions Completed: ☑ YES ☐ NO

Effectiveness Verified: ☑ YES ☐ NO

Verified By: David Thompson, Quality Manager

Date: 15/03/2024

NCR Closed By: David Thompson, Quality Manager

Date: 15/03/2024

LESSONS LEARNED:

1. Calibration management requires systematic approach with automated reminders

2. Operator training must include competency verification

3. Work instructions need built-in verification checkpoints

4. Customer communication protocols must be clearly defined and followed

SYSTEM IMPROVEMENTS IMPLEMENTED:

1. Calibration management software implemented (CAL-SYS-2024)

2. Daily equipment verification checklist introduced

3. Enhanced operator training program with competency matrix

4. Customer communication procedure updated (PROC-CUST-COMM-001)

Note: This revised NCR demonstrates proper root cause analysis, complete corrective/preventive actions, and full closure process per ISO 9001 requirements.