# Root Cause Analysis Report

## METHODOLOGY

This root cause analysis employs the 5-Why technique, Fishbone (Ishikawa) analysis, and Failure Mode and Effects Analysis (FMEA) principles to identify systemic causes of quality failures.

## 1. MATERIAL QUALITY FAILURES

### Issue: 50% Material Rejection Rate (PS-002 & PS-003)

5-Why Analysis:

- Why did we receive incorrect grade material (PS-002)?

  - Supplier shipped Grade 300 instead of Grade 350

- Why did the supplier ship incorrect grade?

  - Purchase order verification process failed at supplier

- Why did the purchase order verification fail?

  - No incoming inspection verification of mill certificates against PO requirements

- Why is there no verification process?

  - Supplier qualification process inadequate - no quality agreement in place

- Why is supplier qualification inadequate?

  - No formal supplier quality management system per ISO 9001 Clause 8.4

Root Cause: Inadequate supplier quality management system and lack of incoming inspection procedures.

Contributing Factors:

- No supplier quality agreements

- Insufficient incoming inspection procedures

- Lack of mill certificate verification protocols

- No supplier performance monitoring

### Issue: Dimensional Non-Conformance (PS-003)

Root Cause Analysis:

- Primary Cause: Supplier cutting/handling processes not controlled

- Secondary Cause: No dimensional verification at goods receipt

- Systemic Cause: Supplier selection criteria do not include process capability assessment

## 2. WELDING PROCEDURE SPECIFICATION FAILURES

### Issue: Multiple AS/NZS 1554.1 Non-Conformances

5-Why Analysis:

- Why does WPS specify 100% CO2 shielding gas?

  - WPS author used AWS standards instead of AS/NZS 1554.1

- Why were AWS standards used instead of Australian standards?

  - Welding engineer training focused on international standards

- Why wasn't AS/NZS 1554.1 compliance verified?

  - No technical review process for WPS development

- Why is there no technical review process?

  - Quality management system lacks document control procedures

- Why are document control procedures inadequate?

  - ISO 9001 Clause 7.5 (Documented Information) not properly implemented

Root Cause: Inadequate document control system and insufficient technical competency in Australian welding standards.

Contributing Factors:

- Lack of AS/NZS 1554.1 specific training

- No technical review process for critical procedures

- Document control system non-compliant with ISO 9001

- No procedure qualification records (PQR) system

### Issue: Incorrect Preheat Temperature Specification

Root Cause Analysis:

- Primary Cause: Welding engineer unfamiliar with AS/NZS 1554.1 preheat requirements for Grade 350 steel

- Secondary Cause: No reference materials or technical library for Australian standards

- Systemic Cause: Competency management system inadequate per ISO 9001 Clause 7.2

## 3. PRODUCTION SCHEDULING FAILURES

### Issue: Resource Allocation Imbalances and Schedule Overruns

5-Why Analysis:

- Why are actual hours consistently exceeding planned hours?

  - Work estimates based on ideal conditions, not realistic scenarios

- Why are estimates unrealistic?

  - No historical data analysis for similar work

- Why is there no historical data analysis?

  - Production planning system lacks data collection and analysis capability

- Why does the planning system lack capability?

  - No continuous improvement process for production planning

- Why is there no continuous improvement?

  - ISO 9001 Clause 10 (Improvement) not effectively implemented

Root Cause: Inadequate production planning system lacking data-driven decision making and continuous improvement.

Contributing Factors:

- No capacity planning for sick leave coverage

- Material procurement not integrated with production scheduling

- No risk assessment for external factors (weather, traffic)

- Lack of performance metrics and KPIs

## 4. CALIBRATION AND MAINTENANCE FAILURES

### Issue: Systematic Calibration Overdue Events

5-Why Analysis:

- Why are calibrations consistently overdue?

  - Calibration scheduling system inadequate

- Why is the scheduling system inadequate?

  - No automated reminder system or calendar integration

- Why is there no automated system?

  - Calibration management treated as administrative task, not critical control

- Why is calibration not treated as critical control?

  - Management understanding of ISO 9001 Clause 7.1.5 requirements insufficient

- Why is management understanding insufficient?

  - No management review of quality system effectiveness per ISO 9001 Clause 9.3

Root Cause: Inadequate management system for monitoring and measuring resources, indicating poor implementation of ISO 9001 requirements.

Contributing Factors:

- No calibration database or tracking system

- Insufficient backup coverage for critical personnel

- Procurement planning not aligned with maintenance schedules

- No risk assessment for equipment downtime

## 5. SAFETY SYSTEM FAILURES

### Issue: Training Gaps and PPE Inadequacy

5-Why Analysis:

- Why was overhead welding safety training not completed?

  - Training matrix doesn't specify position-specific requirements

- Why doesn't training matrix specify position-specific requirements?

  - Job safety analysis (JSA) not conducted for all work positions

- Why wasn't JSA conducted?

  - Safety management system reactive rather than proactive

- Why is safety management reactive?

  - No systematic hazard identification and risk assessment process

- Why is there no systematic process?

  - Safety management system not integrated with quality management system

Root Cause: Inadequate safety management system lacking systematic hazard identification and risk assessment.

Contributing Factors:

- PPE standards not position-specific

- No job safety analysis for specialized work

- Training records not linked to specific job requirements

- Incident investigation process incomplete

## 6. NON-CONFORMANCE MANAGEMENT FAILURES

### Issue: Incomplete NCR Process and Customer Notification Failure

5-Why Analysis:

- Why wasn't customer notification completed?

  - No systematic process for customer communication requirements

- Why is there no systematic process?

  - NCR procedure doesn't clearly define communication responsibilities

- Why doesn't the procedure define responsibilities?

  - Document control system inadequate - procedures not comprehensive

- Why are procedures not comprehensive?

  - No process approach implementation per ISO 9001 Clause 4.4

- Why is process approach not implemented?

  - Quality management system design fundamentally flawed

Root Cause: Fundamental quality management system design flaws indicating poor ISO 9001 implementation.

## SYSTEMIC PATTERNS IDENTIFIED

### 1. Document Control Failures

- Multiple procedures inadequate or missing

- No technical review processes

- Version control issues

- Training on procedures insufficient

### 2. Competency Management Failures

- Training not position-specific

- No competency assessment processes

- Technical knowledge gaps in Australian standards

- No continuous professional development

### 3. Supplier Management Failures

- No supplier quality agreements

- Inadequate supplier qualification

- No performance monitoring

- Poor communication protocols

### 4. Risk Management Failures

- No systematic risk assessment

- Reactive rather than proactive approach

- No contingency planning

- External factors not considered

### 5. Data Management Failures

- No data collection systems

- No performance analysis

- No trend identification

- No data-driven decision making

## INTERCONNECTED FAILURE MODES

The analysis reveals that quality failures are interconnected through:

- Leadership and Management System Failures - Poor implementation of ISO 9001 management principles

- Resource Management Failures - Inadequate control of human resources, infrastructure, and work environment

- Process Control Failures - Lack of systematic approach to process management

- Monitoring and Measurement Failures - Inadequate performance evaluation and improvement systems

These interconnected failures indicate a systemic quality management system breakdown requiring comprehensive corrective action.